Cache Controller Implementation with Write-Through Policy

Group no.10

Group Members:

| Name | Track |
| --- | --- |
| Ehab Mostafa Farghly | Design |
| Mohamed Mahmoud Elfeki | Verification |

Figure 1 RISC-v Architecture

Top level architecture of RISC V with cache memory

Comments:

- Only change we did on RISC-v that we replaced Data Memory Block by our Implementation of Cache Memory.

- The main difference between this architecture and the architecture with cache memory that data memory has 2 more signals stall and memory read.

- stall signal is output from Cache Memory Block and input to the PC. If its value equal 1 it will stall PC and prevent it from incrementing and taking following instructions.

- Memory Read will come from control unit and gives value 1 only when the opcode is LW Instruction.

Top level architecture of cache memory

Figure 2 Cache Memory Block Diagram

Comments:

- This is the Block Diagram representing our implementation of the cache controller.

- DATA Memory & Cache Memory works at the Positive edge clk .

- Cache Controller works at the negative edge clk.

- So, we can consider All Architecture works at negedge clk of RISC-v because the Controlller is the brain of the design.

Cache Controller FSM

Figure 3 Cache Controller FSM

Simulation Testcases & Results

- Test cases of cache memory

/***********************************       testcases     ****************************************/

/* scenario writing in block and check the reading operations of them miss first then all hit */

/** write operations **/

write_data('b111_00010_01 ,'d16  );

write_data('b111_00010_10 ,'d8  );

write_data('b111_00010_00 ,'d4  );

write_data('b111_00010_11 ,'d2  );

/** read operations **/

read_data ('b111_00010_11);

read_data ('b111_00010_10);

read_data ('b111_00010_01);

read_data ('b111_00010_00);

/* scenario writing in block exist in cache and check the writing operations of them hit and store in cache and memory in same time */

/** write operations **/

write_data('b111_00010_01 ,'d100  );

write_data('b111_00010_10 ,'d120  );

/** read operations **/

read_data ('b111_00010_00);

read_data ('b111_00010_01);

read_data ('b111_00010_10);

read_data ('b111_00010_11);

/* scenario addressing same block different in tag with different values */

/** write operations **/

write_data('b101_00010_01 ,'d200  );

write_data('b101_00010_10 ,'d220  );

/** read operations **/

read_data ('b101_00010_00);

read_data ('b101_00010_01);

read_data ('b101_00010_10);

read_data ('b111_00010_11);

read_data ('b111_00010_01);

/*******************  Random processes ******************/

/** write operations **/

write_data('b000_00110_01 ,'d60  );  // Block 6    tag =0  ofsset=1

write_data('b011_01010_01 ,'d100 );  // Block 10   tag =3  offset=1

write_data('b010_00111_01 ,'d20  );  // Block 2    tag=2   offset=1

write_data('b110_10010_10 ,'d60  );  // Blcok 18   tag=6   offset=2                   /*******/

write_data('b000_01011_00 ,'d110 );  // BLock 11   tag =0  offset=0

write_data('b101_10010_11 ,'d200 );  // Block 18   tag=5   offset=3

write_data('b001_00000_11 ,'d1000);  // Block 0    tag =1  offset=3

write_data('b001_10011_01 ,'d190 );  // Block 19   tag =1  offset=1

write_data('b111_11010_10 ,'d260 );  // Block 26   tag =7  offset=2

/** read operations **/

read_data ('b000_00110_01);

read_data ('b011_01010_01);

read_data ('b010_00111_01);

read_data ('b110_10010_10);

read_data ('b110_10010_10);

read_data ('b000_01011_00);

read_data ('b101_10010_11);

read_data ('b001_00000_11);

read_data ('b001_10011_01);

read_data ('b111_11010_10);

Figure 7 Cache simulation Results

Figure 6 Cache simulation Results

Figure 5 Cache simulation Results

Figure 4 Cache simulation Results

Simulation result in this case

- Test cases RISC-v integrated with Cache Memory Block

| Machine Code | Assembly Code |
| --- | --- |
| 00F00093 | addi x1 x0 15 |
| 01400113 | addi x2 x0 20 |
| 01900193 | addi x3 x0 25 |
| 01E00213 | addi x4 x0 30 |
| 01900293 | addi x5 x0 25 |
| 0020A2A3 | sw x2 5(x1) |
| 0030A323 | sw x3 6(x1) |
| 0040A3A3 | sw x4 7(x1) |
| 0050A423 | sw x5 8(x1) |
| 0050A303 | lw x6 5(x1) |
| 0060A383 | lw x7 6(x1) |
| 0070A403 | lw x8 7(x1) |
| 0080A483 | lw x9 8(x1) |

- Simulation result in this case

Figure 9 Reg File Memory

Figure 8 RISC-V with Cache memory simulation Results

Comments:

- As we see in the waveform stall signal is high five times as miss occur in all 4 times of storing and miss occur in first time of loading then hit occur and register files contain the proper results.

RTL of the Cache Memory Controller

Cache Controller

module cache_controller(

input  wire             clk,rst,

input  wire             Mem_Write,Mem_Read,

input  wire             check_Miss_hit,

output reg              write_in_mem,

output reg              write_in_cache,

output reg              move_from_mem,

output reg              move_to_cache,

output reg              read_from_cache,

output reg              stall

);

parameter  IDLE          =3'b000,

READING       =3'b001,

MOVE_TO_CACHE =3'b010,

WRITING       =3'b011,

STALL         =3'b100;

reg     [2:0] current_state,next_state;

reg     [2:0] counter;

reg           counter_enable;

always @(negedge clk or negedge rst)

begin

if(!rst)

begin

counter <='b0;

end

else if (counter_enable)

begin

counter <= counter +'b1;

end

else

begin

counter <= 'b0;

end

end

always @(negedge clk or negedge rst)

begin

if(!rst)

begin

current_state <=IDLE;

end

else

begin

current_state <= next_state;

end

end

always@(*)

begin

write_in_mem    ='b0;

write_in_cache  ='b0;

move_from_mem   ='b0;

move_to_cache   ='b0;

read_from_cache ='b0;

counter_enable  ='b0;

case(current_state)

IDLE        : begin

if((Mem_Read=='b1) && (Mem_Write =='b0) && (check_Miss_hit=='b1) )

begin

next_state = IDLE;

stall           ='b0;

read_from_cache='b1;

end

else if((Mem_Read=='b1) && (Mem_Write =='b0) && (check_Miss_hit=='b0) )

begin

next_state = READING;

stall='b1;

move_from_mem = 'b1;

end

else if((Mem_Read=='b0) && (Mem_Write =='b1) )

begin

next_state = WRITING;

stall='b1;

end

else

begin

next_state =IDLE;

stall='b0;

end

end

READING      : begin

next_state    = MOVE_TO_CACHE;

move_from_mem = 'b1;

move_to_cache = 'b1;

stall         = 'b1;  /*****/

counter_enable= 'b1;

end

MOVE_TO_CACHE  : begin

if(counter == 3'b100)

begin

next_state      = IDLE;

read_from_cache = 'b1;

stall           = 'b0;

counter_enable  = 'b0;

move_from_mem   = 'b0;

move_to_cache   = 'b0;

end

else if(counter == 3'b011)

begin

next_state      = MOVE_TO_CACHE;

read_from_cache = 'b0;

stall           = 'b1;

counter_enable  = 'b1;

move_from_mem   = 'b0;

move_to_cache   = 'b1;

end

else

begin

next_state    = MOVE_TO_CACHE;

move_from_mem = 'b1;

move_to_cache = 'b1;

stall         = 'b1;

counter_enable= 'b1;

end

end

WRITING      : begin

counter_enable='b1;

if(check_Miss_hit )      // 1 hit       0 Miss

begin

write_in_cache ='b1;

write_in_mem   ='b1;

stall          ='b1;

next_state     =STALL;

end

else

begin

write_in_cache ='b0;

write_in_mem   ='b1;

stall          ='b1;

next_state     =STALL;

end

end

STALL      : begin

counter_enable  ='b1;

if(counter == 'b100)

begin

stall='b0;

write_in_cache='b0;

write_in_mem='b0;

next_state =IDLE;

counter_enable  ='b0;

end

else

begin

stall='b1;

write_in_cache='b0;

write_in_mem='b1;

next_state =STALL;

counter_enable  ='b1;

end

end

default     : begin

write_in_mem    ='b0;

write_in_cache  ='b0;

move_from_mem   ='b0;

read_from_cache ='b0;

stall           ='b0;

counter_enable  ='b0;

next_state      =IDLE;

end

endcase

end

endmodule

Cache Memory

module cache_Memory (

input    wire          clk,rst,

input    wire  [9:0]   Word_address,

input    wire  [31:0]  data,

input    wire  [31:0]  Data_IN,

input    wire          write_in_cache,read_from_cache,

input    wire          move_to_cache,

output   reg           check_Miss_hit,

output   reg   [31:0]  Data_Out

);

reg [31:0]  cache_mem [0:127];

reg [3:0]   valid_tag [0:31];

reg [1:0]   move_count;

integer i,k;

always @(posedge clk or negedge rst )

begin

if(!rst)

begin

for (i=0;i<128;i=i+1)

begin

cache_mem[i]<='b0;

end

for (k=0;k<32;k=k+1)

begin

valid_tag[k]<='b0;

end

move_count <= 'b0;

end

else if (write_in_cache)

begin

cache_mem [Word_address[6:0]] <= Data_IN;

valid_tag [Word_address[6:2]] <={1'b1,Word_address[9:7]};

end

else if (move_to_cache)

begin

cache_mem [{Word_address[6:2],move_count}] <= data;

valid_tag [Word_address[6:2]]              <={1'b1,Word_address[9:7]};

move_count                                 <= move_count +'b1;

end

end

always @(*)

begin

if (valid_tag[Word_address[6:2]] == {1'b1 , Word_address [9:7]})

begin

check_Miss_hit ='b1;

end

else

begin

check_Miss_hit ='b0;

end

end

always @(*) begin

if (!rst) begin

Data_Out='b0;

end

else if (read_from_cache)

begin

Data_Out = cache_mem [Word_address[6:0]];

end

else begin

Data_Out='b0;

end

end

endmodule

Data Memory

module  data_memory (

input        wire            clk,rst,

input        wire   [9:0]    Word_address,

input        wire   [31:0]   Data_In,

input        wire            write_in_mem,

input        wire            move_to_cache,

output       reg    [31:0]         data

);

reg [31:0]  main_memory [0:1023];

reg [1:0]   address_count;

integer     i;

always@(posedge clk or negedge rst)

begin

if(!rst)

begin

for(i=0;i<1024;i=i+1)

main_memory[i] <='b0;

address_count  <='b0;

data           <='b0;

end

else

begin

if(write_in_mem)

begin

main_memory[Word_address] <= Data_In;

address_count             <='b0;

end

else if (move_to_cache)

begin

address_count <=address_count+1;

data          <= main_memory[{Word_address[9:2],address_count}];

end

else

begin

address_count <='b0;

end

end

end

endmodule

Cache Memory TOP

module cache_Memory_top (

input  wire          clk,rst,

input  wire          Mem_Write,Mem_Read,

input  wire  [9:0]   Word_address,

input  wire  [31:0]  Data_In,

output wire          stall,

output wire  [31:0]  Data_Out

) ;

/***********************************/

wire         check_Miss_hit;

wire         write_in_mem,write_in_cache;

wire         move_from_mem;

wire         move_to_cache;

wire         read_from_cache;

cache_controller u0(

.clk(clk),

.rst(rst),

.Mem_Write(Mem_Write),

.Mem_Read(Mem_Read),

.check_Miss_hit(check_Miss_hit),

.write_in_mem(write_in_mem),

.write_in_cache(write_in_cache),

.move_from_mem(move_from_mem),

.move_to_cache(move_to_cache),

.read_from_cache(read_from_cache),

.stall(stall)

);

/************************************/

wire  [31:0]     data;

cache_Memory u1(

.clk(clk),

.rst(rst),

.Word_address(Word_address),

.data(data),

.Data_IN(Data_In),

.write_in_cache(write_in_cache),

.read_from_cache(read_from_cache),

.move_to_cache(move_to_cache),

.check_Miss_hit(check_Miss_hit),

.Data_Out(Data_Out)

);

/************************************/

data_memory u2(

.clk(clk),

.rst(rst),

.Word_address(Word_address),

.Data_In(Data_In),

.write_in_mem(write_in_mem),

.move_to_cache(move_from_mem),

.data(data)

);

endmodule